/************************************************************************************

*	LCD Connection:

*

*   CS			PA.7 chip select

*   C/D(RS) 	PA.6 command/data mode

*   WR 		PA.5 write strobe

*   RD 		PA.4 read strobe

*   RST 		PF.4 reset

*   BKLT 		PF.3 backlight

*   DATA 		PB data line 8-15 on pins B.0-B.7

*   XP+ 		PE.2 touch X+	AIN1

*   YP+ 		PE.3 touch Y+	AIN0

*   XN- 		PA.3 touch X-

*   YN- 		PA.2 touch Y-

*

*

*	SHT75 - Temperature and Humidity sensor

*	SDA		PD.7

*	SCL		PD.6

*

*  BMP085 - Pressure and Altitude sensor

*  SDA		PE.5

*  SCL		PE.4

*

*  Anemometer - Wind speed

*  Resistor divider 5k		PD.2

*

*  Wind direction

*  Resistor divider 10k		PE1			AIN2

*

*  	if(wvalue <= 552) strcpy(pole, "W_");

else if(wvalue <= 940) strcpy(pole, "NW");

else if(wvalue <= 1472) strcpy(pole, "N_");

else if(wvalue <= 2127) strcpy(pole, "SW");

else if(wvalue <= 2748) strcpy(pole, "NE");

else if(wvalue <= 3232) strcpy(pole, "S_");

else if(wvalue <= 3535) strcpy(pole, "SE");

else if(wvalue <= 4096) strcpy(pole, "E_");

else strcpy(pole, "W_");

*

*	Rain Gauge		PD.3

*

*  LED display		PC7		Tx3 serial

*

*  Light Sensor	PE.0	AIN3

*

*

* Nazvoslovi

* ----------------------

* tlak 				- Pressure

* srazky 				- WaterLevel

* rosny bod 			- Dewpoint

* pocitova teplota 	- ApparentTemperature

* vyska 				- AltitudeMy

* intenzita osvetleni 	- Light

* vitr 				- Wind		(smer vetru - wind direction, rychlost vetru - wind speed)

* datum a cas 			- Date

* ikona pocasi 		- Image

* teplota uvnitr 		- TemperatureIn

* teplota venku 		- Temperature

* vlhkost 				- Humidity

*

*

***************************************************************************************/

#include "inc/hw_gpio.h"

#include "inc/hw_ints.h"

#include "inc/hw_memmap.h"

#include "inc/hw_nvic.h"

#include "inc/hw_ssi.h"

#include "inc/hw_sysctl.h"

#include "inc/hw_types.h"

#include "driverlib/debug.h"

#include "driverlib/fpu.h"

#include "driverlib/flash.h"

#include "driverlib/gpio.h"

#include "driverlib/i2c.h"

#include "driverlib/interrupt.h"

#include "driverlib/pin_map.h"

#include "driverlib/rom.h"

#include "driverlib/rom_map.h"

#include "driverlib/ssi.h"

#include "driverlib/sysctl.h"

#include "driverlib/systick.h"

#include "driverlib/timer.h"

#include "driverlib/uart.h"

#include "driverlib/udma.h"

#include "driverlib/adc.h"

#include "grlib/grlib.h"

#include "grlib/widget.h"

#include "grlib/canvas.h"

#include "grlib/checkbox.h"

#include "grlib/container.h"

#include "grlib/pushbutton.h"

#include "grlib/radiobutton.h"

#include "grlib/slider.h"

#include "utils/uartstdio.h"

#include "utils/ustdlib.h"

#include "drivers/Adafruit320x240x16TouchTFT_ILI9325.h"

//#include "drivers/sound.h"

#include "drivers/touch.h"

#include "images.h"

#include "sht75my.h"

#include "math.h"

#include "I2C_Stellaris_API.h"

#include "bmp085my.h"

#include <string.h>

#include <stdlib.h>

#define BMP085_ADDR		0x77

#define PI 3.1415

// odkaz na pole s obrazkem

extern const unsigned char g_pucImage[];

tContext sContext;

tRectangle sRect;

void ClrScreen(void);

//*****************************************************************************

// The error routine that is called if the driver library encounters an error.

//*****************************************************************************

#ifdef DEBUG

void

__error__(char *pcFilename, unsigned long ulLine)

{

}

#endif

//*****************************************************************************

//

// The DMA control structure table.

//

//*****************************************************************************

#ifdef ewarm

#pragma data_alignment=1024

tDMAControlTable sDMAControlTable[64];

#elif defined(ccs)

#pragma DATA_ALIGN(sDMAControlTable, 1024)

tDMAControlTable sDMAControlTable[64];

#else

tDMAControlTable sDMAControlTable[64] __attribute__ ((aligned(1024)));

#endif

//   ----------------------------------------------------------------------------------------------------------------------------------------------------------------   PROMENNE

unsigned int humidity = 0;

unsigned int temperature = 0;

float temperature_f = 0.0;

float temperature_f_max = 0.0;

float temperature_f_min = 100.0;

int temperature_i = 0;

float temperature_app = 0.0;

float humidity_f = 0.0;

float humidity_real = 0.0;

float humidity_real_max = 0.0;

float humidity_real_min = 100.0;

float wvp_exp = 0.0;

float wvp = 0.0;

float dewpoint = 0.0;

float dewa = 0.0;

float dewb = 0.0;

char string_t[5];				// text temperature

char string_t_max[5];			// text temperature_max

char string_t_min[5];			// text temperature_min

char string_t_a[5];			//text temperature apparent

char string_h[5];				// text humidity

char string_h_i[2];				// text humidity int

char string_h_max[5];			// text humidity_i_max

char string_h_min[5];			// text humidity_i_min

char string_d[5];				// text dew point

char string_tl[5];				// text tlak

char string_v[5];				// text vyska

char string_w[5];				// text water level in ml/h

char string_w_max[5];			// text maximum water level in ml/h

char string_time[8] = "00:00:00";			// text time

char string_time_hour[2] = "00";		// text time hour

unsigned int time_hour = 0; 	// time + 2 hour for CZ time

unsigned char poleTempSend[] = "1234";		//pole pro odeslani na LED displej

int sendChar = 0;

unsigned int msTimer = 0;

volatile unsigned int pocet = 0;

volatile unsigned int pocet_preklopeni = 0;		//Preklopeni pri 0,2794 mm -> 294 mL na m2

volatile unsigned int pocet_preklopeni_hour = 0;

volatile unsigned int overeni = 10;

volatile unsigned int overeni2 = 10;

float otacky_f = 0.0;

float otacky_f_max = 0.0;

char string_r[5];

char string_r_kopie[5];

char string_r_max[5];

char *beafourt_table[] = {"bezvetri","vanek","vetrik","slaby vitr","mirny vitr","cerstvy vitr","silny vitr","mirny vichr","cerstvy vichr","silny vichr","plny vichr","vichrice","orkan"};		//Beafourtova stupnice sily vetru

// 0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12 - 17

// < 0.5, ~ 1.25, ~ 3, ~ 5, ~ 7, ~ 9.5, ~ 12, ~ 14.5, ~ 17.5, ~ 21, ~ 24.5, ~ 29, > 30

int poleTemp[300];

int poleTempIndex = 0;

int beafourt = 0;

long pressure = 0;

double altitude_my = 0;

int circle_x = 279, circle_y = 40, circle_rad = 35;

unsigned long ulADC0_Value[4];		//hodnota z AIN0

unsigned int wvalue = 0;

char pole[] = "00";

double smer = 0;

unsigned int vteriny = 0;

float water_level = 0.0;

float water_level_max = 0.0;

int obnoveni_displeje = 0;	//obnoveni displeje s textem

int kalibrace = 1;			//kalibrace displeje

int bmp085_enable = 1;		//BMP085 enable

unsigned long polozkaMenu = 0;

char znakGps;								// GPS

char poleGpsRx[12] = "------123456";

unsigned int a = 0;

// --------------------------------------------------------------------------------------------- Deklarace funkci

void zakladniGrafika();

void CallPressure();

void CallWaterLevel();

void CallDewpoint();

void CallApparentTemperature();

void CallAltitudeMy();

void CallLight();

void CallWind();

void CallDate();

void CallImage();

void CallTemperatureIn();

void CallTemperature();

void CallHumidity();

void BackButtonPressure();

void BackButtonWaterLevel();

void BackButtonDewpoint();

void BackButtonApparentTemperature();

void BackButtonAltitudeMy();

void BackButtonLight();

void BackButtonWind();

void BackButtonDate();

void BackButtonImage();

void BackButtonTemperatureIn();

void BackButtonTemperature();

void BackButtonHumidity();

extern tCanvasWidget canMain;					// hlavni Canvas

extern tCanvasWidget canButtons;				// Canvas tlacitka

extern tCanvasWidget canPressure;				// Canvas Pressure

extern tCanvasWidget canWaterLevel;				// Canvas WaterLevel

extern tCanvasWidget canDewpoint;				// Canvas Dewpoint

extern tCanvasWidget canApparentTemperature;	// Canvas ApparentTemperature

extern tCanvasWidget canAltitudeMy;				// Canvas AltitudeMy

extern tCanvasWidget canLight;					// Canvas Light

extern tCanvasWidget canWind;					// Canvas Wind

extern tCanvasWidget canDate;					// Canvas Date

extern tCanvasWidget canImage;					// Canvas Image

extern tCanvasWidget canTemperatureIn;			// Canvas TemperatureIn

extern tCanvasWidget canTemperature;			// Canvas Temperature

extern tCanvasWidget canHumidity;				// Canvas Humidity

extern tPushButtonWidget widBackBtnPressure;

extern tPushButtonWidget widBackBtnWaterLevel;

extern tPushButtonWidget widBackBtnDewpoint;

extern tPushButtonWidget widBackBtnApparentTemperature;

extern tPushButtonWidget widBackBtnAltitudeMy;

extern tPushButtonWidget widBackBtnLight;

extern tPushButtonWidget widBackBtnWind;

extern tPushButtonWidget widBackBtnDate;

extern tPushButtonWidget widBackBtnImage;

extern tPushButtonWidget widBackBtnTemperatureIn;

extern tPushButtonWidget widBackBtnTemperature;

extern tPushButtonWidget widBackBtnHumidity;

extern tPushButtonWidget widPushBtnPressure;

extern tPushButtonWidget widPushBtnWaterLevel;

extern tPushButtonWidget widPushBtnDewpoint;

extern tPushButtonWidget widPushBtnApparentTemperature;

extern tPushButtonWidget widPushBtnAltitudeMy;

extern tPushButtonWidget widPushBtnLight;

extern tPushButtonWidget widPushBtnWind;

extern tPushButtonWidget widPushBtnDate;

extern tPushButtonWidget widPushBtnImage;

extern tPushButtonWidget widPushBtnTemperatureIn;

extern tPushButtonWidget widPushBtnTemperature;

extern tPushButtonWidget widPushBtnHumidity;

// -------------------------------------------------------------------------------------------------------------------------------------------------------------------------- CANVAS

//*****************************************************************************

// Panel 13 - Humidity

Canvas(canHumidity, &canMain, 0, &widBackBtnHumidity, &g_sAdafruit320x240x16_ILI9325, 0, 0, 320, 240, CANVAS_STYLE_FILL, 0, 0, 0, 0, 0, 0, 0);

RectangularButton(widBackBtnHumidity, &canHumidity, 0, 0, &g_sAdafruit320x240x16_ILI9325, 0, 0, 320, 240, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, BackButtonHumidity);

//*****************************************************************************

// Panel 12 - Temperature

Canvas(canTemperature, &canMain, 0, &widBackBtnTemperature, &g_sAdafruit320x240x16_ILI9325, 0, 0, 320, 240, CANVAS_STYLE_FILL, 0, 0, 0, 0, 0, 0, 0);

RectangularButton(widBackBtnTemperature, &canTemperature, 0, 0, &g_sAdafruit320x240x16_ILI9325, 0, 0, 320, 240, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, BackButtonTemperature);

//*****************************************************************************

// Panel 11 - TemperatureIn

Canvas(canTemperatureIn, &canMain, 0, &widBackBtnTemperatureIn, &g_sAdafruit320x240x16_ILI9325, 0, 0, 320, 240, CANVAS_STYLE_FILL, 0, 0, 0, 0, 0, 0, 0);

RectangularButton(widBackBtnTemperatureIn, &canTemperatureIn, 0, 0, &g_sAdafruit320x240x16_ILI9325, 0, 0, 320, 240, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, BackButtonTemperatureIn);

//*****************************************************************************

// Panel 10 - Image

Canvas(canImage, &canMain, 0, &widBackBtnImage, &g_sAdafruit320x240x16_ILI9325, 0, 0, 320, 240, CANVAS_STYLE_FILL, 0, 0, 0, 0, 0, 0, 0);

RectangularButton(widBackBtnImage, &canImage, 0, 0, &g_sAdafruit320x240x16_ILI9325, 0, 0, 320, 240, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, BackButtonImage);

//*****************************************************************************

// Panel 9 - Date

Canvas(canDate, &canMain, 0, &widBackBtnDate, &g_sAdafruit320x240x16_ILI9325, 0, 0, 320, 240, CANVAS_STYLE_FILL, 0, 0, 0, 0, 0, 0, 0);

RectangularButton(widBackBtnDate, &canDate, 0, 0, &g_sAdafruit320x240x16_ILI9325, 0, 0, 320, 240, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, BackButtonDate);

//*****************************************************************************

// Panel 8 - Wind

Canvas(canWind, &canMain, 0, &widBackBtnWind, &g_sAdafruit320x240x16_ILI9325, 0, 0, 320, 240, CANVAS_STYLE_FILL, 0, 0, 0, 0, 0, 0, 0);

RectangularButton(widBackBtnWind, &canWind, 0, 0, &g_sAdafruit320x240x16_ILI9325, 0, 0, 320, 240, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, BackButtonWind);

//*****************************************************************************

// Panel 7 - Light

Canvas(canLight, &canMain, 0, &widBackBtnLight, &g_sAdafruit320x240x16_ILI9325, 0, 0, 320, 240, CANVAS_STYLE_FILL, 0, 0, 0, 0, 0, 0, 0);

RectangularButton(widBackBtnLight, &canLight, 0, 0, &g_sAdafruit320x240x16_ILI9325, 0, 0, 320, 240, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, BackButtonLight);

//*****************************************************************************

// Panel 6 - AltitudeMy

Canvas(canAltitudeMy, &canMain, 0, &widBackBtnAltitudeMy, &g_sAdafruit320x240x16_ILI9325, 0, 0, 320, 240, CANVAS_STYLE_FILL, 0, 0, 0, 0, 0, 0, 0);

RectangularButton(widBackBtnAltitudeMy, &canAltitudeMy, 0, 0, &g_sAdafruit320x240x16_ILI9325, 0, 0, 320, 240, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,BackButtonAltitudeMy);

//*****************************************************************************

// Panel 5 - ApparentTemperature

Canvas(canApparentTemperature, &canMain, 0, &widBackBtnApparentTemperature, &g_sAdafruit320x240x16_ILI9325, 0, 0, 320, 240, CANVAS_STYLE_FILL, 0, 0, 0, 0, 0, 0, 0);

RectangularButton(widBackBtnApparentTemperature, &canApparentTemperature, 0, 0, &g_sAdafruit320x240x16_ILI9325, 0, 0, 320, 240, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,BackButtonApparentTemperature);

//*****************************************************************************

// Panel 4 - GraphDewpoint

Canvas(canDewpoint, &canMain, 0, &widBackBtnDewpoint, &g_sAdafruit320x240x16_ILI9325, 0, 0, 320, 240, CANVAS_STYLE_FILL, 0, 0, 0, 0, 0, 0, 0);

RectangularButton(widBackBtnDewpoint, &canDewpoint, 0, 0, &g_sAdafruit320x240x16_ILI9325, 0, 0, 320, 240, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,BackButtonDewpoint);

//*****************************************************************************

// Panel 3 - GraphWaterLevel

Canvas(canWaterLevel, &canMain, 0, &widBackBtnWaterLevel, &g_sAdafruit320x240x16_ILI9325, 0, 0, 320, 240, CANVAS_STYLE_FILL, 0, 0, 0, 0, 0, 0, 0);

RectangularButton(widBackBtnWaterLevel, &canWaterLevel, 0, 0, &g_sAdafruit320x240x16_ILI9325, 0, 0, 320, 240, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,BackButtonWaterLevel);

//*****************************************************************************

// Panel 2 - GraphPressure

Canvas(canPressure, &canMain, 0, &widBackBtnPressure, &g_sAdafruit320x240x16_ILI9325, 0, 0, 320, 240, CANVAS_STYLE_FILL, 0, 0, 0, 0, 0, 0, 0);

RectangularButton(widBackBtnPressure, &canPressure, 0, 0, &g_sAdafruit320x240x16_ILI9325, 0, 0, 320, 240, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, BackButtonPressure);

//*****************************************************************************

// Panel 1 - Hlavni s tlacitky

Canvas(canButtons, &canMain, 0, &widPushBtnPressure, &g_sAdafruit320x240x16_ILI9325, 0, 0, 320, 240, CANVAS_STYLE_FILL, ClrBlack, 0, 0, 0, 0, 0, 0);

RectangularButton(widPushBtnPressure, &canButtons, &widPushBtnWaterLevel, 0, &g_sAdafruit320x240x16_ILI9325, 1, 2, 59, 78, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, CallPressure);

RectangularButton(widPushBtnWaterLevel, &canButtons, &widPushBtnDewpoint, 0, &g_sAdafruit320x240x16_ILI9325, 1, 81, 59, 79, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, CallWaterLevel);

RectangularButton(widPushBtnDewpoint, &canButtons, &widPushBtnApparentTemperature, 0, &g_sAdafruit320x240x16_ILI9325, 1, 161, 119, 39, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, CallDewpoint);

RectangularButton(widPushBtnApparentTemperature, &canButtons, &widPushBtnAltitudeMy, 0, &g_sAdafruit320x240x16_ILI9325, 1, 201, 119, 38, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, CallApparentTemperature);

RectangularButton(widPushBtnAltitudeMy, &canButtons, &widPushBtnLight, 0, &g_sAdafruit320x240x16_ILI9325, 121, 161, 118, 39, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, CallAltitudeMy);

RectangularButton(widPushBtnLight, &canButtons, &widPushBtnWind, 0, &g_sAdafruit320x240x16_ILI9325, 121, 201, 118, 38,  0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, CallLight);

RectangularButton(widPushBtnWind, &canButtons, &widPushBtnDate, 0, &g_sAdafruit320x240x16_ILI9325, 240, 2, 79, 138, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, CallWind);

RectangularButton(widPushBtnDate, &canButtons, &widPushBtnImage, 0, &g_sAdafruit320x240x16_ILI9325, 240, 141, 79, 98,  0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, CallDate);

RectangularButton(widPushBtnImage, &canButtons, &widPushBtnTemperatureIn, 0, &g_sAdafruit320x240x16_ILI9325, 181, 2, 58, 78,  0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, CallImage);

RectangularButton(widPushBtnTemperatureIn, &canButtons, &widPushBtnTemperature, 0, &g_sAdafruit320x240x16_ILI9325, 61, 2, 119, 78, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, CallTemperatureIn);

RectangularButton(widPushBtnTemperature, &canButtons, &widPushBtnHumidity, 0, &g_sAdafruit320x240x16_ILI9325, 61, 81, 109, 79,  0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, CallTemperature);

RectangularButton(widPushBtnHumidity, &canButtons, 0, 0, &g_sAdafruit320x240x16_ILI9325, 171, 81, 68, 79,  0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, CallHumidity);

//*****************************************************************************

// Panel 0 - Hlavni Canvas

Canvas(canMain, WIDGET_ROOT, 0, 0, &g_sAdafruit320x240x16_ILI9325, 0, 0, 320, 240, CANVAS_STYLE_FILL, ClrBlack, 0, 0, 0, 0, 0, 0);

//   -----------------------------------------------------------------------------------------------------------------------------------------------------------------------   FUNKCE

//convert float to ascii for serial print

void ftoamy2(long double f,char *buf,int dplaces)

{

int pos=0,ix,dp,num,loop;

if (f<0)

{

buf[pos++]='-';

f = -f;

}

dp=0;

loop = dplaces + 2;

while (f>=10.0)

{

f=f/10.0;

dp++;

loop++;

}

//    for (ix=1;ix<8;ix++)

for (ix=1;ix<loop;ix++)

{

num = f;

f=f-num;

if (num>9)

buf[pos++]='#';

else

buf[pos++]='0'+num;

if (dp==0) buf[pos++]='.';

f=f*10.0;

dp--;

}

buf[pos] = '\0'; // null term

}

void itoa( int n, unsigned char * s )

{

int j,i = 0;

unsigned char tmp;

if( n < 0 )                                                 // if negativ set - sign

{  s[0] = '-'; s++;

n = -n;

}

do                                                          // calculate next digit

{  s[i++] = n%10 + '0';

n /= 10;

} while( n > 0 );

for( j = 0; j < i/2 ; ++j )                                 // revert string

{  tmp = s[j];

s[j] = s[i-j-1];

s[i-j-1] = tmp;

}

s[i] = '\0';

}

// Smazani obrazovky LCD

void ClrScreen()

{

sRect.sXMin = 0;

sRect.sYMin = 0;

sRect.sXMax = 319;

sRect.sYMax = 239;

GrContextForegroundSet(&sContext, ClrBlack);

GrRectFill(&sContext, &sRect);

GrFlush(&sContext);

}

// Delay function in miliseconds

void DelayMs(unsigned long ulSeconds)

{

// Loop while there are more seconds to wait.

while(ulSeconds--)

{

// Wait until the SysTick value is less than 1000.

while(SysTickValueGet() > 1000)

{

}

// Wait until the SysTick value is greater than 1000.

while(SysTickValueGet() < 1000)

{

}

}

}

//Interrupt for anemometer and rain gauge

void IntHandler()

{

static unsigned int lastTime = 0;

if(GPIOPinIntStatus(GPIO_PORTD_BASE, true) == GPIO_PIN_2 && (msTimer - lastTime >= 10))

{

lastTime = msTimer;

//Delay(10);

overeni = GPIOPinRead(GPIO_PORTD_BASE, GPIO_PIN_2);

if(overeni == 0)

{

pocet++;

}

}

// Rain Gauge

static unsigned int lastTime2 = 0;

if(GPIOPinIntStatus(GPIO_PORTD_BASE, true) == GPIO_PIN_3 && (msTimer - lastTime2 >= 10))

{

lastTime2 = msTimer;

overeni2 = GPIOPinRead(GPIO_PORTD_BASE, GPIO_PIN_3);

if(overeni2 == 0)

{

pocet_preklopeni++;

UARTprintf("Preklopeni v preruseni: %d \n", pocet_preklopeni);

}

}

GPIOPinIntClear(GPIO_PORTD_BASE, GPIO_PIN_2);

GPIOPinIntClear(GPIO_PORTD_BASE, GPIO_PIN_3);

}

// Interrupt Timer

void SysTickIntHandler()

{

msTimer++;

if(msTimer % 1000 == 0)

{

//UARTprintf("Pocet otacek = %d", pocet/2);	//2 pulse per round

//2.4 km/h = 0.666 m/s, = 1 otacka

otacky_f = (pocet/2) * 0.6666666;

if ((otacky_f >= 0.0)&&(otacky_f < 0.5)) beafourt = 0;	//Urovne beafourta

if ((otacky_f > 0.5)&&(otacky_f < 1.25)) beafourt = 1;

if ((otacky_f > 1.25)&&(otacky_f < 3.0)) beafourt =   2;

if ((otacky_f > 3.0)&&(otacky_f < 5.0)) beafourt = 3;

if ((otacky_f > 5.0)&&(otacky_f < 7.0)) beafourt =   4;

if ((otacky_f > 7.0)&&(otacky_f < 9.5)) beafourt = 5;

if ((otacky_f > 9.5)&&(otacky_f < 12.0)) beafourt =   6;

if ((otacky_f > 12.0)&&(otacky_f < 14.5)) beafourt = 7;

if ((otacky_f > 14.5)&&(otacky_f < 17.5)) beafourt =   8;

if ((otacky_f > 17.5)&&(otacky_f < 21.0)) beafourt = 9;

if ((otacky_f > 21.0)&&(otacky_f < 24.5)) beafourt =   10;

if ((otacky_f > 24.5)&&(otacky_f < 29.0)) beafourt = 11;

if (otacky_f > 30.0) beafourt =   12;

if (otacky_f_max < otacky_f) otacky_f_max = otacky_f;  // ulozeni maxima otacek

ftoamy2(otacky_f_max, string_r_max, 2);

string_r_kopie[0] = string_r[0];

string_r_kopie[1] = string_r[1];

string_r_kopie[2] = string_r[2];

string_r_kopie[3] = string_r[3];

string_r_kopie[4] = string_r[4];

//Conver float do char with 2 decimal point

ftoamy2(otacky_f, string_r, 2);

pocet = 0;

//Water level

vteriny++;

//	UARTprintf("Vterina = : %d ", vteriny);

if(vteriny > 3599)

{

pocet_preklopeni_hour = pocet_preklopeni;

pocet_preklopeni = 0;

vteriny = 0;

}

}

}

void zakladniGrafika()

{

//bily ramecek

sRect.sXMin = 0;

sRect.sYMin = 0;

sRect.sXMax = 319;

sRect.sYMax = 239;

GrContextForegroundSet(&sContext, ClrWhite);

GrRectDraw(&sContext, &sRect);

//rozdelovaci cary

GrContextForegroundSet(&sContext, ClrWhite);

GrLineDraw(&sContext, 60, 0, 60, 160);		//1

GrLineDraw(&sContext, 0, 160, 239, 160);	//2

GrLineDraw(&sContext, 239, 0, 239, 238);	//3

GrLineDraw(&sContext, 0, 200, 239, 200);	//4

GrLineDraw(&sContext, 120, 160, 120, 238);	//5

GrLineDraw(&sContext, 0, 80, 239, 80);		//6

GrLineDraw(&sContext, 239, 140, 318, 140);	//7

GrLineDraw(&sContext, 180, 0, 180, 80);		//8

GrLineDraw(&sContext, 170, 80, 170, 160);	//9

}

void bilyRamecek()

{

//ClrScreen();

sRect.sXMin = 0;

sRect.sYMin = 0;

sRect.sXMax = 319;

sRect.sYMax = 239;

GrContextForegroundSet(&sContext, ClrWhite);

GrRectDraw(&sContext, &sRect);

}

void bileOsy()

{

GrLineDrawH(&sContext, 10, 310, 210);	// osa x

GrLineDrawV(&sContext, 10, 10, 210);	// osa y

}

//--------------------------------------------------------- Widgets funkce

void Hlavni(tWidget *pWidget)

{

zakladniGrafika();

obnoveni_displeje = 1;

}

void BackButtonPressure()

{

WidgetRemove((tWidget *)&widBackBtnPressure);

WidgetRemove((tWidget *)&canPressure);

WidgetAdd(WIDGET_ROOT, (tWidget *)&canButtons);

WidgetPaint(WIDGET_ROOT);

WidgetMessageQueueProcess();

zakladniGrafika();

obnoveni_displeje = 1;

}

void BackButtonWaterLevel()

{

WidgetRemove((tWidget *)&widBackBtnWaterLevel);

WidgetRemove((tWidget *)&canWaterLevel);

WidgetAdd(WIDGET_ROOT, (tWidget *)&canButtons);

WidgetPaint(WIDGET_ROOT);

WidgetMessageQueueProcess();

zakladniGrafika();

obnoveni_displeje = 1;

}

void BackButtonDewpoint()

{

WidgetRemove((tWidget *)&widBackBtnDewpoint);

WidgetRemove((tWidget *)&canDewpoint);

WidgetAdd(WIDGET_ROOT, (tWidget *)&canButtons);

WidgetPaint(WIDGET_ROOT);

WidgetMessageQueueProcess();

zakladniGrafika();

obnoveni_displeje = 1;

}

void BackButtonApparentTemperature()

{

WidgetRemove((tWidget *)&widBackBtnApparentTemperature);

WidgetRemove((tWidget *)&canApparentTemperature);

WidgetAdd(WIDGET_ROOT, (tWidget *)&canButtons);

WidgetPaint(WIDGET_ROOT);

WidgetMessageQueueProcess();

zakladniGrafika();

obnoveni_displeje = 1;

}

void BackButtonAltitudeMy()

{

WidgetRemove((tWidget *)&widBackBtnAltitudeMy);

WidgetRemove((tWidget *)&canAltitudeMy);

WidgetAdd(WIDGET_ROOT, (tWidget *)&canButtons);

WidgetPaint(WIDGET_ROOT);

WidgetMessageQueueProcess();

zakladniGrafika();

obnoveni_displeje = 1;

}

void BackButtonLight()

{

WidgetRemove((tWidget *)&widBackBtnLight);

WidgetRemove((tWidget *)&canLight);

WidgetAdd(WIDGET_ROOT, (tWidget *)&canButtons);

WidgetPaint(WIDGET_ROOT);

WidgetMessageQueueProcess();

zakladniGrafika();

obnoveni_displeje = 1;

}

void BackButtonWind()

{

WidgetRemove((tWidget *)&widBackBtnWind);

WidgetRemove((tWidget *)&canWind);

WidgetAdd(WIDGET_ROOT, (tWidget *)&canButtons);

WidgetPaint(WIDGET_ROOT);

WidgetMessageQueueProcess();

zakladniGrafika();

obnoveni_displeje = 1;

}

void BackButtonDate()

{

WidgetRemove((tWidget *)&widBackBtnDate);

WidgetRemove((tWidget *)&canDate);

WidgetAdd(WIDGET_ROOT, (tWidget *)&canButtons);

WidgetPaint(WIDGET_ROOT);

WidgetMessageQueueProcess();

zakladniGrafika();

obnoveni_displeje = 1;

}

void BackButtonImage()

{

WidgetRemove((tWidget *)&widBackBtnImage);

WidgetRemove((tWidget *)&canImage);

WidgetAdd(WIDGET_ROOT, (tWidget *)&canButtons);

WidgetPaint(WIDGET_ROOT);

WidgetMessageQueueProcess();

zakladniGrafika();

obnoveni_displeje = 1;

}

void BackButtonTemperatureIn()

{

WidgetRemove((tWidget *)&widBackBtnTemperatureIn);

WidgetRemove((tWidget *)&canTemperatureIn);

WidgetAdd(WIDGET_ROOT, (tWidget *)&canButtons);

WidgetPaint(WIDGET_ROOT);

WidgetMessageQueueProcess();

zakladniGrafika();

obnoveni_displeje = 1;

}

void BackButtonTemperature()

{

WidgetRemove((tWidget *)&widBackBtnTemperature);

WidgetRemove((tWidget *)&canTemperature);

WidgetAdd(WIDGET_ROOT, (tWidget *)&canButtons);

WidgetPaint(WIDGET_ROOT);

WidgetMessageQueueProcess();

zakladniGrafika();

obnoveni_displeje = 1;

}

void BackButtonHumidity()

{

WidgetRemove((tWidget *)&widBackBtnHumidity);

WidgetRemove((tWidget *)&canHumidity);

WidgetAdd(WIDGET_ROOT, (tWidget *)&canButtons);

WidgetPaint(WIDGET_ROOT);

WidgetMessageQueueProcess();

zakladniGrafika();

obnoveni_displeje = 1;

}

void CallPressure()

{

obnoveni_displeje = 0;

WidgetRemove((tWidget *)&canButtons);

WidgetAdd(WIDGET_ROOT, (tWidget *)&canPressure);

WidgetAdd(WIDGET_ROOT, (tWidget *)&widBackBtnPressure);

WidgetPaint(WIDGET_ROOT);

WidgetMessageQueueProcess();

bilyRamecek();

bileOsy();

GrContextFontSet(&sContext, &g_sFontCm20);

GrStringDraw(&sContext, "Pressure hPa/hours", -1, 80, 215, 1);

GrContextForegroundSet(&sContext, ClrAqua);		//vykresleni nahodnych hodnot do grafu

int xx;

for(xx = 11; xx < 311; xx++)

{

GrLineDrawV(&sContext, xx, poleTemp[xx-11], 209);

}

}

void CallWaterLevel()	// vodni sloupce

{

obnoveni_displeje = 0;

WidgetRemove((tWidget *)&canButtons);

WidgetAdd(WIDGET_ROOT, (tWidget *)&canWaterLevel);

WidgetAdd(WIDGET_ROOT, (tWidget *)&widBackBtnWaterLevel);

WidgetPaint(WIDGET_ROOT);

WidgetMessageQueueProcess();

bilyRamecek();

GrLineDrawH(&sContext, 12, 44, 210);

GrLineDrawH(&sContext, 56, 88, 210);

GrLineDrawH(&sContext, 100, 132, 210);

GrLineDrawH(&sContext, 144, 176, 210);

GrLineDrawH(&sContext, 188, 220, 210);

GrLineDrawH(&sContext, 232, 264, 210);

GrLineDrawH(&sContext, 276, 308, 210);

GrLineDrawV(&sContext, 12, 20, 210);

GrLineDrawV(&sContext, 44, 20, 210);

GrLineDrawV(&sContext, 56, 20, 210);

GrLineDrawV(&sContext, 88, 20, 210);

GrLineDrawV(&sContext, 100, 20, 210);

GrLineDrawV(&sContext, 132, 20, 210);

GrLineDrawV(&sContext, 144, 20, 210);

GrLineDrawV(&sContext, 176, 20, 210);

GrLineDrawV(&sContext, 188, 20, 210);

GrLineDrawV(&sContext, 220, 20, 210);

GrLineDrawV(&sContext, 232, 20, 210);

GrLineDrawV(&sContext, 264, 20, 210);

GrLineDrawV(&sContext, 276, 20, 210);

GrLineDrawV(&sContext, 308, 20, 210);

sRect.sXMin = 13;	// prvni sloupec

sRect.sYMin = 209;

sRect.sXMax = 43;

sRect.sYMax = 100;	//vyska = 210 - yMax

GrContextForegroundSet(&sContext, ClrLightBlue);

GrRectFill(&sContext, &sRect);

sRect.sXMin = 57;	// druhy sloupec

sRect.sYMin = 209;

sRect.sXMax = 87;

sRect.sYMax = 123;	//vyska = 210 - yMax

GrContextForegroundSet(&sContext, ClrLightBlue);

GrRectFill(&sContext, &sRect);

sRect.sXMin = 145;	// ctvrty sloupec

sRect.sYMin = 209;

sRect.sXMax = 175;

sRect.sYMax = 60;	//vyska = 210 - yMax

GrContextForegroundSet(&sContext, ClrLightBlue);

GrRectFill(&sContext, &sRect);

sRect.sXMin = 233;	// sesty sloupec

sRect.sYMin = 209;

sRect.sXMax = 263;

sRect.sYMax = 200;	//vyska = 210 - yMax

GrContextForegroundSet(&sContext, ClrLightBlue);

GrRectFill(&sContext, &sRect);

GrContextForegroundSet(&sContext, ClrWhite);

GrContextFontSet(&sContext, &g_sFontCm20);

GrStringDraw(&sContext, "Po", -1, 20, 215, 1);

GrStringDraw(&sContext, "Ut", -1, 64, 215, 1);

GrStringDraw(&sContext, "St", -1, 108, 215, 1);

GrStringDraw(&sContext, "Ct", -1, 152, 215, 1);

GrStringDraw(&sContext, "Pa", -1, 196, 215, 1);

GrStringDraw(&sContext, "So", -1, 240, 215, 1);

GrStringDraw(&sContext, "Ne", -1, 284, 215, 1);

}

void CallDewpoint()

{

obnoveni_displeje = 0;

WidgetRemove((tWidget *)&canButtons);

WidgetAdd(WIDGET_ROOT, (tWidget *)&canDewpoint);

WidgetAdd(WIDGET_ROOT, (tWidget *)&widBackBtnDewpoint);

WidgetPaint(WIDGET_ROOT);

WidgetMessageQueueProcess();

bilyRamecek();

bileOsy();

GrContextFontSet(&sContext, &g_sFontCm20);

GrStringDraw(&sContext, "Dewpoint C/hours", -1, 80, 215, 1);

GrContextForegroundSet(&sContext, ClrLightGreen);		//vykresleni nahodnych hodnot do grafu

int xx;

for(xx = 11; xx < 310; xx++)

{

GrLineDrawV(&sContext, xx, (209-(rand() % 100)), 209);

}

}

void CallApparentTemperature()

{

obnoveni_displeje = 0;

WidgetRemove((tWidget *)&canButtons);

WidgetAdd(WIDGET_ROOT, (tWidget *)&canApparentTemperature);

WidgetAdd(WIDGET_ROOT, (tWidget *)&widBackBtnApparentTemperature);

WidgetPaint(WIDGET_ROOT);

WidgetMessageQueueProcess();

bilyRamecek();

bileOsy();

GrContextFontSet(&sContext, &g_sFontCm20);

GrStringDraw(&sContext, "Apparent Temperature C/hours", -1, 20, 215, 1);

GrContextForegroundSet(&sContext, ClrGreenYellow);		//vykresleni nahodnych hodnot do grafu

int xx;

for(xx = 11; xx < 310; xx++)

{

GrLineDrawV(&sContext, xx, (209-(rand() % 100)), 209);

}

}

void CallAltitudeMy()

{

obnoveni_displeje = 0;

WidgetRemove((tWidget *)&canButtons);

WidgetAdd(WIDGET_ROOT, (tWidget *)&canAltitudeMy);

WidgetAdd(WIDGET_ROOT, (tWidget *)&widBackBtnAltitudeMy);

WidgetPaint(WIDGET_ROOT);

WidgetMessageQueueProcess();

bilyRamecek();

GrContextFontSet(&sContext, &g_sFontCm20);

GrStringDraw(&sContext, "AltitudeMy", -1, 20, 50, 1);

}

void CallLight()

{

obnoveni_displeje = 0;

WidgetRemove((tWidget *)&canButtons);

WidgetAdd(WIDGET_ROOT, (tWidget *)&canLight);

WidgetAdd(WIDGET_ROOT, (tWidget *)&widBackBtnLight);

WidgetPaint(WIDGET_ROOT);

WidgetMessageQueueProcess();

bilyRamecek();

GrContextFontSet(&sContext, &g_sFontCm26b);

GrStringDraw(&sContext, "Light min = 156 lux", -1, 20, 50, 1);

GrStringDraw(&sContext, "Light now = 453 lux", -1, 20, 100, 1);

GrStringDraw(&sContext, "Light max = 968 lux", -1, 20, 150, 1);

}

void CallWind()

{

obnoveni_displeje = 0;

WidgetRemove((tWidget *)&canButtons);

WidgetAdd(WIDGET_ROOT, (tWidget *)&canWind);

WidgetAdd(WIDGET_ROOT, (tWidget *)&widBackBtnWind);

WidgetPaint(WIDGET_ROOT);

WidgetMessageQueueProcess();

bilyRamecek();

//kruh pro smer vetru

GrContextForegroundSet(&sContext, ClrViolet);

GrCircleDraw(&sContext, 160, 120, 70);

//	vykresleni cary smeru vetru

GrContextForegroundSet(&sContext, ClrLightPink);

double alfa = 0;

double beta = 0;

alfa = ((cos(smer)*70)+160);

beta =  ((-1)*sin(smer)*70)+120;

GrLineDraw(&sContext, 160, 120, alfa, beta);

GrContextFontSet(&sContext, &g_sFontCm20);

GrStringDraw(&sContext, "Wind max =  9,7 m/s", -1, 40, 215, 1);

}

void CallDate()

{

obnoveni_displeje = 0;

WidgetRemove((tWidget *)&canButtons);

WidgetAdd(WIDGET_ROOT, (tWidget *)&canDate);

WidgetAdd(WIDGET_ROOT, (tWidget *)&widBackBtnDate);

WidgetPaint(WIDGET_ROOT);

WidgetMessageQueueProcess();

bilyRamecek();

GrContextFontSet(&sContext, &g_sFontCm30);

GrStringDraw(&sContext, "year   2013", -1, 20, 50, 1);

GrStringDraw(&sContext, "date   18.8", -1, 20, 80, 1);

GrStringDraw(&sContext, "time   19:02:54", -1, 20, 110, 1);

GrStringDraw(&sContext, "gps N  49°12'41.861", -1, 20, 140, 1);

GrStringDraw(&sContext, "gps E  17°38'45.607", -1, 20, 170, 1);

}

void CallImage()

{

obnoveni_displeje = 0;

WidgetRemove((tWidget *)&canButtons);

WidgetAdd(WIDGET_ROOT, (tWidget *)&canImage);

WidgetAdd(WIDGET_ROOT, (tWidget *)&widBackBtnImage);

WidgetPaint(WIDGET_ROOT);

WidgetMessageQueueProcess();

bilyRamecek();

GrContextFontSet(&sContext, &g_sFontCm30i);

GrStringDraw(&sContext, "Meteostanice", -1, 40, 50, 1);

GrStringDraw(&sContext, "verze: 2013-08-19", -1, 40, 100, 1);

}

void CallTemperatureIn()

{

obnoveni_displeje = 0;

WidgetRemove((tWidget *)&canButtons);

WidgetAdd(WIDGET_ROOT, (tWidget *)&canTemperatureIn);

WidgetAdd(WIDGET_ROOT, (tWidget *)&widBackBtnTemperatureIn);

WidgetPaint(WIDGET_ROOT);

WidgetMessageQueueProcess();

bilyRamecek();

bileOsy();

GrContextFontSet(&sContext, &g_sFontCm20);

GrStringDraw(&sContext, "Temperature In C/hours", -1, 80, 215, 1);

GrContextForegroundSet(&sContext, ClrLightGreen);		//vykresleni nahodnych hodnot do grafu

int xx;

for(xx = 11; xx < 310; xx++)

{

GrLineDrawV(&sContext, xx, (209-(rand() % 100)), 209);

}

}

void CallTemperature()

{

obnoveni_displeje = 0;

WidgetRemove((tWidget *)&canButtons);

WidgetAdd(WIDGET_ROOT, (tWidget *)&canTemperature);

WidgetAdd(WIDGET_ROOT, (tWidget *)&widBackBtnTemperature);

WidgetPaint(WIDGET_ROOT);

WidgetMessageQueueProcess();

bilyRamecek();

bileOsy();

GrContextFontSet(&sContext, &g_sFontCm20);

GrStringDraw(&sContext, "Temperature C/hours", -1, 80, 215, 1);

GrContextForegroundSet(&sContext, ClrRed);		//vykresleni nahodnych hodnot do grafu

int xx;

for(xx = 11; xx < 310; xx++)

{

GrLineDrawV(&sContext, xx, (209-(rand() % 100)), 209);

}

}

void CallHumidity()

{

obnoveni_displeje = 0;

WidgetRemove((tWidget *)&canButtons);

WidgetAdd(WIDGET_ROOT, (tWidget *)&canHumidity);

WidgetAdd(WIDGET_ROOT, (tWidget *)&widBackBtnHumidity);

WidgetPaint(WIDGET_ROOT);

WidgetMessageQueueProcess();

bilyRamecek();

bileOsy();

GrContextFontSet(&sContext, &g_sFontCm20);

GrStringDraw(&sContext, "Humidity %/hours", -1, 80, 215, 1);

GrContextForegroundSet(&sContext, ClrYellow);		//vykresleni nahodnych hodnot do grafu

int xx;

for(xx = 11; xx < 310; xx++)

{

GrLineDrawV(&sContext, xx, (209-(rand() % 100)), 209);

}

}

//   -------------------------------------------------------------------------------------------------------------------------------------------------------------------------   MAIN

int main(void)

{

//  tContext sContext2;

// Save FPU Registers before Interrupt calling

ROM_FPULazyStackingEnable();

// Configure the system clock to run at 40MHz

SysCtlClockSet(SYSCTL_SYSDIV_5|SYSCTL_USE_PLL|SYSCTL_XTAL_16MHZ|SYSCTL_OSC_MAIN);

// Set up a SysTick Interrupt 1000 per second

SysTickPeriodSet(SysCtlClockGet() / 1000);

SysTickIntEnable();

SysTickEnable();

// SHT75

//	Configure pins PD6 and PD7 as output for SHT75

//First open the lock and select the bits we want to modify in the GPIO commit register.

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOD);

HWREG(GPIO_PORTD_BASE + GPIO_O_LOCK) = GPIO_LOCK_KEY_DD;

HWREG(GPIO_PORTD_BASE + GPIO_O_CR) = 0x80;

GPIOPinTypeGPIOOutput(GPIO_PORTD_BASE, GPIO_PIN_6 | GPIO_PIN_7);

GPIOPinWrite(GPIO_PORTD_BASE, GPIO_PIN_7, GPIO_PIN_7);

GPIOPinWrite(GPIO_PORTD_BASE, GPIO_PIN_6, 0);

// Anemometer PD2 and Rain Gauge PD3

// Enable the GPIO port D for interrupt

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOD);

// Configures pin(s) for use as GPIO inputs PD2 - INPUT for Anemometer

GPIOPinTypeGPIOInput(GPIO_PORTD_BASE, GPIO_PIN_2 | GPIO_PIN_3);

// PadConfigureSet - for PULL-UP (is insert in GPIOPinTypeInput only as PUSH-PULL)

GPIOPadConfigSet(GPIO_PORTD_BASE, GPIO_PIN_2 | GPIO_PIN_3, GPIO_STRENGTH_2MA, GPIO_PIN_TYPE_STD_WPU);

//	GPIOPadConfigSet(GPIO_PORTD_BASE, GPIO_PIN_3, GPIO_STRENGTH_2MA, GPIO_PIN_TYPE_STD_WPU);

// Define edge of pin for interrupt

GPIOIntTypeSet(GPIO_PORTD_BASE, GPIO_PIN_2 | GPIO_PIN_3, GPIO_FALLING_EDGE);

// Enable Interrupt for pin

GPIOPinIntEnable(GPIO_PORTD_BASE, GPIO_PIN_2 | GPIO_PIN_3);

// Enable interrupt for PORT D, IRQ number

IntEnable(19);

// Enable Interrupt

IntMasterEnable();

// BMP085

//I2C0 enable and run

I2CSetup(I2C2_MASTER_BASE, 0);

ROM_IntMasterEnable();

// Wind direction PE1

SysCtlPeripheralEnable(SYSCTL_PERIPH_ADC0);		//povolení ADC0

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOE);	//povolení portu E

GPIOPinTypeADC(GPIO_PORTE_BASE, GPIO_PIN_1 );	//povolení pinu 1

SysCtlADCSpeedSet(SYSCTL_ADCSPEED_250KSPS);		//rychlost vzorkování

//	ADCHardwareOversampleConfigure(ADC0_BASE, 64);	//přidáno, HW průměrování

ADCSequenceDisable(ADC0_BASE,1);				//vypnutí ADC před konfigurací

ADCSequenceConfigure(ADC0_BASE, 1, ADC_TRIGGER_PROCESSOR, 0);	// konfigurace ADC: ADC0, SS2, Trigger - CPU, Priority - 0 (high)

ADCSequenceStepConfigure(ADC0_BASE, 1, 0, ADC_CTL_CH0);		//konfigurace sequenceru 3 pro kanal 0, povolene preruseni

ADCSequenceStepConfigure(ADC0_BASE, 1, 1, ADC_CTL_CH1);		//konfigurace sequenceru 3 pro kanal 0, povolene preruseni

ADCSequenceStepConfigure(ADC0_BASE, 1, 2, ADC_CTL_CH2);		//konfigurace sequenceru 3 pro kanal 0, povolene preruseni

ADCSequenceStepConfigure(ADC0_BASE, 1, 3, ADC_CTL_CH3 | ADC_CTL_IE | ADC_CTL_END);		//konfigurace sequenceru 3 pro kanal 0, povolene preruseni

ADCSequenceEnable(ADC0_BASE, 1);	//povolení ADC0 Sequenceru SS3

ADCIntClear(ADC0_BASE, 1);		//vymazani priznaku preruseni

//port F for LED

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOF);

GPIOPinTypeGPIOOutput(GPIO_PORTF_BASE, GPIO_PIN_1|GPIO_PIN_2|GPIO_PIN_3);

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_1|GPIO_PIN_2, 0x00);

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_3, 0xff);

// Initialize the UART.

ROM_SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOA);

ROM_GPIOPinConfigure(GPIO_PA0_U0RX);

ROM_GPIOPinConfigure(GPIO_PA1_U0TX);

ROM_GPIOPinTypeUART(GPIO_PORTA_BASE, GPIO_PIN_0 | GPIO_PIN_1);

UARTStdioInit(0);

// START

UARTprintf("START\n");

// Initialize UART 3

//

// Enable Peripheral Clocks

//

SysCtlPeripheralEnable(SYSCTL_PERIPH_UART3);

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOC);

//

// Enable port PC6 for UART3 U3RX

//

GPIOPinConfigure(GPIO_PC6_U3RX);

GPIOPinTypeUART(GPIO_PORTC_BASE, GPIO_PIN_6);

//

// Enable port PC7 for UART3 U3TX

//

GPIOPinConfigure(GPIO_PC7_U3TX);

GPIOPinTypeUART(GPIO_PORTC_BASE, GPIO_PIN_7);

//

// Initialize the UART. Set the baud rate, number of data bits, turn off

// parity, number of stop bits, and stick mode.

//

UARTConfigSetExpClk(UART3_BASE, SysCtlClockGet(), 115200,

(UART_CONFIG_WLEN_8 | UART_CONFIG_STOP_ONE |

UART_CONFIG_PAR_NONE));

//

// Enable the UART.

//

UARTEnable(UART3_BASE);

//get calibration data from BMP085

if(bmp085_enable == 1) bmp_get_cal();

// Inicializace LCD

Adafruit320x240x16_ILI9325Init();

GrContextInit(&sContext, &g_sAdafruit320x240x16_ILI9325);

ClrScreen();

// Initialize the graphics context.

//GrContextInit(&sContext, &g_sAdafruit320x240x16_ILI9325);

SysCtlPeripheralEnable(SYSCTL_PERIPH_UDMA);

SysCtlDelay(10);

uDMAControlBaseSet(&sDMAControlTable[0]);

uDMAEnable();

if(kalibrace == 1)

{

// Initialize the touch screen driver and have it route its messages to the widget tree.

TouchScreenInit();

// Paint touch calibration targets and collect calibration data

GrContextForegroundSet(&sContext, ClrWhite);

GrContextBackgroundSet(&sContext, ClrBlack);

GrContextFontSet(&sContext, &g_sFontCm20);

GrStringDraw(&sContext, "Tukni na kolecka pro kalibraci", -1, 0, 0, 1);

GrCircleDraw(&sContext, 32, 34, 10);

GrFlush(&sContext);

TouchScreenCalibrationPoint(32, 34, 0);

GrCircleDraw(&sContext, 280, 200, 10);

GrFlush(&sContext);

TouchScreenCalibrationPoint(280, 200, 1);

GrCircleDraw(&sContext, 200, 40, 10);

GrFlush(&sContext);

TouchScreenCalibrationPoint(200, 40, 2);

// Calculate and set calibration matrix

long* plCalibrationMatrix = TouchScreenCalibrate();

// Enable touch screen event handler for grlib widgets

TouchScreenCallbackSet(WidgetPointerMessage);

}

ClrScreen();

TouchScreenCallbackSet(WidgetPointerMessage);

WidgetAdd(WIDGET_ROOT, (tWidget *)&canMain);

WidgetRemove((tWidget *)&widBackBtnPressure);

WidgetRemove((tWidget *)&widBackBtnWaterLevel);

WidgetRemove((tWidget *)&widBackBtnDewpoint);

WidgetRemove((tWidget *)&widBackBtnApparentTemperature);

WidgetRemove((tWidget *)&widBackBtnAltitudeMy);

WidgetRemove((tWidget *)&widBackBtnLight);

WidgetRemove((tWidget *)&widBackBtnWind);

WidgetRemove((tWidget *)&widBackBtnDate);

WidgetRemove((tWidget *)&widBackBtnImage);

WidgetRemove((tWidget *)&widBackBtnTemperatureIn);

WidgetRemove((tWidget *)&widBackBtnTemperature);

WidgetRemove((tWidget *)&widBackBtnHumidity);

WidgetPaint(WIDGET_ROOT);

WidgetMessageQueueProcess();

WidgetAdd(WIDGET_ROOT, (tWidget *)&canButtons);

WidgetPaint(WIDGET_ROOT);

WidgetMessageQueueProcess();

zakladniGrafika();

obnoveni_displeje = 1;

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_1|GPIO_PIN_2|GPIO_PIN_3, 0x00);	//zhasnutí všech LED diod

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_3, 0xff);	//zapne LCD podsvit

//   --------------------------------------------------------------------------------------------------------------------------------------------------------------------   WHILE CYCLE

while(1)

{

//		nahodne = rand() % 100;

WidgetMessageQueueProcess();	//vykresleni widgetu

//Pressure

pressure = bmp_get_value_pres();

pressure = pressure / 100;

altitude_my = bmp_get_value_alt();

ftoamy2(pressure, string_tl, 2);

ftoamy2(altitude_my, string_v, 2);

// Temperature

resetSHT(); //reset SHT75

startSHT();    //start sequence from sensor datasheet

writeByteSHT(0x03);

temperature = readByteSHT();

temperature_f = -40.1 + (0.01 * temperature);

if (temperature_f_max < temperature_f) temperature_f_max = temperature_f;  // ulozeni maxima teploty

ftoamy2(temperature_f_max, string_t_max, 2);

if (temperature_f_min > temperature_f) temperature_f_min = temperature_f;  // ulozeni minima teploty

ftoamy2(temperature_f_min, string_t_min, 2);

//Convert float do char with 2 decimal point

ftoamy2(temperature_f, string_t, 2);

//		UARTprintf("Temperature = %s C   ", string_t);

temperature_i = (temperature_f*100);	//temperature_i pro LED displej

if((temperature_i < 1000)&&(temperature_i > 100))

{

itoa(temperature_i, poleTempSend+1);

poleTempSend[0] = '0';

}

if(temperature_i < 100)

{

itoa(temperature_i, poleTempSend+2);

poleTempSend[0] = '0';

poleTempSend[1] = '0';

}

if(temperature_i > 1000) itoa(temperature_i, poleTempSend);			//naplni pole hodnotou temperature_i

for(sendChar = 0; sendChar<4;sendChar++)	//odesila znaky z pole pro LED displej

{

UARTCharPut(UART3_BASE, poleTempSend[sendChar]);

}

if(poleTempIndex > 300) poleTempIndex = 0;

poleTemp[poleTempIndex] = temperature_i/100;

poleTempIndex++;

//		DelayMs(100);

// Humidity

resetSHT(); //reset SHT75

startSHT();    //start sequence from sensor datasheet

writeByteSHT(0x05);

humidity = readByteSHT();

humidity_f = -2.0468 + (0.0367 * humidity) + ((-1.5955*1e-6)*humidity*humidity) ;     //vypocet linearni vlhkosti

humidity_real = (temperature_f - 25) * (0.01 + (0.00008 * humidity)) + humidity_f;

if (humidity_real_max < humidity_real) humidity_real_max = humidity_real;  // ulozeni maxima humidity

ftoamy2(humidity_real_max, string_h_max, 2);

if (humidity_real_min > humidity_real) humidity_real_min = humidity_real;  // ulozeni minima humidity

ftoamy2(humidity_real_min, string_h_min, 2);

//Conver float do char with 2 decimal point

ftoamy2(humidity_real, string_h, 2);

string_h_i[0] = string_h[0];

string_h_i[1] = string_h[1];

//		UARTprintf("Humidity = %s %%   ", string_h);

// Dew point

dewa = log(humidity_real/100);

dewb = (17.62 * temperature_f) / (243.12 + temperature_f);            //vypocet rosneho bodu

dewpoint = 243.12 * ((dewa + dewb)/(17.62 - dewa - dewb));

ftoamy2(dewpoint, string_d, 2);

//		UARTprintf("Dew point = %s ", string_d);

//Apparent temperature

//Water vapour pressure wvp

wvp_exp = (temperature_f/(237.7+temperature_f))*17.27;

wvp_exp = exp(wvp_exp);

wvp = (humidity_real/100)*wvp_exp;

temperature_app = temperature_f+(0.33*wvp)-(0.7*otacky_f)-4;

ftoamy2(temperature_app, string_t_a, 2);

// Wind direction

ADCIntClear(ADC0_BASE, 1);	//clear interrupt for ADC0

ADCProcessorTrigger(ADC0_BASE, 1);	// Trigger od CPU

while(!ADCIntStatus(ADC0_BASE, 1, false))	//cekani na konverzi dat od ADC0

{

}

ADCSequenceDataGet(ADC0_BASE, 1, ulADC0_Value);	//precteni dat

//		UARTprintf("   AIN2(PE1) = %4d   ", ulADC0_Value[2]);

wvalue = ulADC0_Value[2];

if(wvalue <= 552) {strcpy(pole, "W_"); smer = 0;}

else if(wvalue <= 940) {strcpy(pole, "NW"); smer = (7*PI/4);}

else if(wvalue <= 1472) {strcpy(pole, "N_"); smer = (6*PI/4);}

else if(wvalue <= 2127) {strcpy(pole, "SW"); smer = (PI/4);}

else if(wvalue <= 2748) {strcpy(pole, "NE"); smer = (5*PI/4);}

else if(wvalue <= 3232) {strcpy(pole, "S_"); smer = (2*PI/4);}

else if(wvalue <= 3535) {strcpy(pole, "SE"); smer = (3*PI/4);}

else if(wvalue <= 4096) {strcpy(pole, "E_"); smer = PI;}

else {strcpy(pole, "W_"); smer = 0;}

//		UARTprintf("%s\n", pole);

// Water Level

//	UARTprintf("   pocet preklopeni hour: %d", pocet_preklopeni_hour);

water_level = pocet_preklopeni_hour * 0.2794;

ftoamy2(water_level, string_w, 4);

//	UARTprintf("   Water level: = %s \n", string_w);

if (water_level_max < water_level) water_level_max = water_level;  // ulozeni maxima vody

ftoamy2(water_level_max, string_w_max, 4);

//  ----------------------------------------------  GRAFIKA

if(obnoveni_displeje == 1)

{

//Smazani mista pro smerovku vetru

sRect.sXMin = 240;

sRect.sYMin = 2;

sRect.sXMax = 317;

sRect.sYMax = 118;

GrContextForegroundSet(&sContext, ClrBlack);

GrRectFill(&sContext, &sRect);

//kruh pro smer vetru

GrContextForegroundSet(&sContext, ClrPink);

GrCircleDraw(&sContext, 279, 40, 35);

//	vykresleni cary smeru vetru

GrContextForegroundSet(&sContext, ClrLightPink);

double alfa = 0;

double beta = 0;

alfa = ((cos(smer) * circle_rad)+circle_x);

beta =  (     (-1)   *sin(smer)   *circle_rad)   +circle_y  ;

GrLineDraw(&sContext, circle_x, circle_y, alfa,   beta            );

//Smazani mista pro cisla rychlosti vetru

sRect.sXMin = 240;

sRect.sYMin = 85;

sRect.sXMax = 294;

sRect.sYMax = 105;

GrContextForegroundSet(&sContext, ClrBlack);

GrRectFill(&sContext, &sRect);

//Text rychlost vetru

GrContextForegroundSet(&sContext, ClrPink);

GrContextFontSet(&sContext, &g_sFontCmss20b);

GrStringDraw(&sContext, string_r, -1, 243, 85, 0);

GrContextFontSet(&sContext, &g_sFontCmss12b);

GrStringDraw(&sContext, "m/s", -1, 291, 92, 0);

//Smazani mista pro cisla maximalni rychlosti vetru

sRect.sXMin = 240;

sRect.sYMin = 105;

sRect.sXMax = 270;

sRect.sYMax = 119;

GrContextForegroundSet(&sContext, ClrBlack);

GrRectFill(&sContext, &sRect);

//Text rychlost vetru

GrContextForegroundSet(&sContext, ClrPink);

GrContextFontSet(&sContext, &g_sFontCmss16b);

GrStringDraw(&sContext, string_r_max, -1, 243, 105, 0);

GrContextFontSet(&sContext, &g_sFontCmss12b);

GrStringDraw(&sContext, "m/s", -1, 281, 107, 0);

//Smazani mista pro Beafourtova stupnici

sRect.sXMin = 240;

sRect.sYMin = 120;

sRect.sXMax = 315;

sRect.sYMax = 139;

GrContextForegroundSet(&sContext, ClrBlack);

GrRectFill(&sContext, &sRect);

//Text rychlost vetru

GrContextForegroundSet(&sContext, ClrPink);

GrContextFontSet(&sContext, &g_sFontCmss16b);

GrStringDraw(&sContext, beafourt_table[beafourt], -1, 243, 125, 0);

//Smazani mista pro cisla teploty

sRect.sXMin = 65;

sRect.sYMin = 90;

sRect.sXMax = 140;

sRect.sYMax = 115;

GrContextForegroundSet(&sContext, ClrBlack);

GrRectFill(&sContext, &sRect);

//Text s teplotou

GrContextForegroundSet(&sContext, ClrRed);

GrContextFontSet(&sContext, &g_sFontCmss30b);

GrStringDraw(&sContext, string_t, -1, 65, 90, 0);

GrStringDraw(&sContext, "C", -1, 140, 90, 0);

//Smazani mista pro cisla teploty_max

sRect.sXMin = 65;

sRect.sYMin = 120;

sRect.sXMax = 110;

sRect.sYMax = 135;

GrContextForegroundSet(&sContext, ClrBlack);

GrRectFill(&sContext, &sRect);

//Text s teplotou_max

GrContextForegroundSet(&sContext, ClrRed);

GrContextFontSet(&sContext, &g_sFontCmss12b);

GrStringDraw(&sContext, string_t_max, -1, 65, 120, 0);

GrStringDraw(&sContext, "C", -1, 100, 120, 0);

//Smazani mista pro cisla teploty_min

sRect.sXMin = 65;

sRect.sYMin = 135;

sRect.sXMax = 110;

sRect.sYMax = 150;

GrContextForegroundSet(&sContext, ClrBlack);

GrRectFill(&sContext, &sRect);

//Text s teplotou_min

GrContextForegroundSet(&sContext, ClrRed);

GrContextFontSet(&sContext, &g_sFontCmss12b);

GrStringDraw(&sContext, string_t_min, -1, 65, 135, 0);

GrStringDraw(&sContext, "C", -1, 100, 135, 0);

//Smazani mista pro cisla vlhkosti

sRect.sXMin = 175;

sRect.sYMin = 90;

sRect.sXMax = 210;

sRect.sYMax = 115;

GrContextForegroundSet(&sContext, ClrBlack);

GrRectFill(&sContext, &sRect);

//Text s vlhkosti

GrContextForegroundSet(&sContext, ClrYellow);

GrContextFontSet(&sContext, &g_sFontCmss30b);

GrStringDraw(&sContext, string_h_i, -1, 175, 90, 0);

GrStringDraw(&sContext, "%", -1, 210, 90, 0);

//Smazani mista pro cisla vlhkosti_max

sRect.sXMin = 175;

sRect.sYMin = 120;

sRect.sXMax = 230;

sRect.sYMax = 135;

GrContextForegroundSet(&sContext, ClrBlack);

GrRectFill(&sContext, &sRect);

//Text s vlhkosti_max

GrContextForegroundSet(&sContext, ClrYellow);

GrContextFontSet(&sContext, &g_sFontCmss12b);

GrStringDraw(&sContext, string_h_max, -1, 175, 120, 0);

GrStringDraw(&sContext, "%", -1, 210, 120, 0);

//Smazani mista pro cisla vlhkosti_min

sRect.sXMin = 175;

sRect.sYMin = 135;

sRect.sXMax = 230;

sRect.sYMax = 150;

GrContextForegroundSet(&sContext, ClrBlack);

GrRectFill(&sContext, &sRect);

//Text s vlhkosti_min

GrContextForegroundSet(&sContext, ClrYellow);

GrContextFontSet(&sContext, &g_sFontCmss12b);

GrStringDraw(&sContext, string_h_min, -1, 175, 135, 0);

GrStringDraw(&sContext, "%", -1, 210, 135, 0);

//Smazani mista pro rosny bod

sRect.sXMin = 10;

sRect.sYMin = 170;

sRect.sXMax = 115;

sRect.sYMax = 195;

GrContextForegroundSet(&sContext, ClrBlack);

GrRectFill(&sContext, &sRect);

//Text s rosnym bodem

GrContextForegroundSet(&sContext, ClrLightGreen);

GrContextFontSet(&sContext, &g_sFontCmss30b);

GrStringDraw(&sContext, string_d, -1, 10, 170, 0);

GrStringDraw(&sContext, "C", -1, 90, 170, 0);

//Smazani mista pro tlak

sRect.sXMin = 5;

sRect.sYMin = 5;

sRect.sXMax = 55;

sRect.sYMax = 25;

GrContextForegroundSet(&sContext, ClrBlack);

GrRectFill(&sContext, &sRect);

//Text s tlakem

GrContextForegroundSet(&sContext, ClrAliceBlue);

GrContextFontSet(&sContext, &g_sFontCmss12b);

GrStringDraw(&sContext, string_tl, -1, 5, 5, 0);

GrStringDraw(&sContext, "hPa", -1, 15, 15, 0);

//Smazani mista pro nadmorskou vysku

sRect.sXMin = 130;

sRect.sYMin = 170;

sRect.sXMax = 235;

sRect.sYMax = 195;

GrContextForegroundSet(&sContext, ClrBlack);

GrRectFill(&sContext, &sRect);

//Text s nadmorskou vyskou

GrContextForegroundSet(&sContext, ClrMoccasin);

GrContextFontSet(&sContext, &g_sFontCmss24b);

GrStringDraw(&sContext, string_v, -1, 130, 170, 0);

GrContextForegroundSet(&sContext, ClrMoccasin);

GrContextFontSet(&sContext, &g_sFontCmss12b);

GrStringDraw(&sContext, "m.n.m.", -1, 200, 178, 0);

//Smazani mista pro zdanlivou teplotu

sRect.sXMin = 10;

sRect.sYMin = 205;

sRect.sXMax = 115;

sRect.sYMax = 235;

GrContextForegroundSet(&sContext, ClrBlack);

GrRectFill(&sContext, &sRect);

//Text se zdanlivou teplotou

GrContextForegroundSet(&sContext, ClrGreenYellow);

GrContextFontSet(&sContext, &g_sFontCmss30b);

GrStringDraw(&sContext, string_t_a, -1, 10, 205, 0);

GrStringDraw(&sContext, "C", -1, 90, 205, 0);

//Smazani mista pro mnozstvi srazek

sRect.sXMin = 5;

sRect.sYMin = 85;

sRect.sXMax = 55;

sRect.sYMax = 105;

GrContextForegroundSet(&sContext, ClrBlack);

GrRectFill(&sContext, &sRect);

//Text s mnozstvim srazek

GrContextForegroundSet(&sContext, ClrAqua);

GrContextFontSet(&sContext, &g_sFontCmss12b);

GrStringDraw(&sContext, string_w, -1, 5, 85, 0);

GrStringDraw(&sContext, "mm/h", -1, 15, 95, 0);

//Smazani mista pro maximum mnozstvi srazek

sRect.sXMin = 5;

sRect.sYMin = 115;

sRect.sXMax = 55;

sRect.sYMax = 135;

GrContextForegroundSet(&sContext, ClrBlack);

GrRectFill(&sContext, &sRect);

//Text s maximem mnozstvi srazek

GrContextForegroundSet(&sContext, ClrAqua);

GrContextFontSet(&sContext, &g_sFontCmss12b);

GrStringDraw(&sContext, string_w_max, -1, 5, 115, 0);

GrStringDraw(&sContext, "mm/h", -1, 15, 125, 0);

//Smazani mista pro hodiny

sRect.sXMin = 240;

sRect.sYMin = 142;

sRect.sXMax = 318;

sRect.sYMax = 238;

GrContextForegroundSet(&sContext, ClrBlack);

GrRectFill(&sContext, &sRect);

//Text s hodinami

GrContextForegroundSet(&sContext, ClrGainsboro);

GrContextFontSet(&sContext, &g_sFontCmss20b);

GrStringDraw(&sContext, string_time, 8, 242, 142, 0);

}

}

}