ACTIVE COOLING SINGLE FAMILY RESIDENTIAL HOUSE IN INDONESIA

HVAC System

The HVAC systems uses 2 ductwork for supply air and return air, which are for the supply air  uses rectangle duct and for the return air uses round duct. Both ducting connected to the Air Handling Unit (AHU) that functionate to manage airflow to the conditioned space. Some of the return air will be go back to heat pump and the other will deliver to Variable Air Volume (VAV) Box as the fresh air. The ratio of these air are 0.4, means 2 of 5 of return air will be recirculated as supply air after being filtered in AHU. Meanwhile the rest will be continued into heat pump then to the water tank for heating the water.

The cooling source used in this HVAC system is air source reversible heat pump. As a cooler this pump assumed work only 12 hours a day in average. Since the heat pump is reversible it is also capable produce heat, and this waste heat is optimized for heating the domestic water in the house. For the electric source, the heat pump directly connected into the electric panel as well as into the main control cabinet (panel) in order being part of the building integrated system.

Supply air duct itself has automatic damper on each branch. It could regulate the airflow amount into the room based on thermostat controllers which are located in the every conditioned room. All these distributed controls are connected to the sensor and will ignite A/C based on signal received.

Ductwork airflow systems

Ductwork systems in 3D view

Damper (yellow) in air supply duct

Ductwork and Water Piping System in Ceiling View

Heat pump, Water Tank and water solar heater placed above garage

Electrical System

Common residential house in Indonesia still using utility grid with 220 V single phase 2 wire. For power supply could be vary categories based on the price per kWh between 1300VA, 2200VA, 3500VA -5500VA, more than 6600VA. In this model, the house use 2200VA from utility grid and 1100VA from the photovoltaic panel.

The electrical panel connected to power meter from utility grid and DC to AC inverter as PV panel source. The output would supply control cabinet before reach the load.

Roof View with PV Panel

Receptacle Circuit Systems

Power meter and DC to AC inverter

Electric panel and Control cabinet located inside garage

Security System

The house using alarm system at the main gate, sprinkle system as fire protection, CCTV and motion sensor to avoid unwanted intruder, but for advance execution it should be done manually. Advance execution means the owner must call the police or the fire fighter in case of worst thing happen because the emergency call in Indonesia does not have integration communication to the residential single family house. For now it is only possible for the public building or apartement.

Plumbing and Sprinkle Systems

Inside hall with lighting at night

Lighting Inside House at Night

The view of patio when sensor activated at night

Communication system

The communication for building automation system used by distribution mini control in each room which contains of lighting (except in all bedrooms), cooling (HVAC), hot water system, electrical power metering and IP-CCTV (security), fire alarm system.

For the building management system (BMS) based on BACnet ASHRAE/ANSI standard communication protocol for building automation applied to this house.

The Communication Devices (Router) Inside House

Inside bedroom with the controller device

CCTV reachable area

CCTV reachable area top view

House top view

Kitchen view

Bathroom with light sensor inside